Technical Manual

Grid Congestion Analysis Pipeline

Nikola Momchilov | Fontys Innovation Lab | 2026

# 1. Purpose

This manual provides step-by-step instructions for reproducing the grid congestion analysis. It is written for Desislav (continuing next semester) and future researchers at Fontys Innovation Lab.

# 2. System Requirements

| Software | Version | Purpose | Download |
| --- | --- | --- | --- |
| Python | 3.11+ | Notebooks | python.org |
| SNAP Desktop | 11 | Sentinel-2 preprocessing | step.esa.int |
| Streamlit | 1.30+ | Web application | pip install streamlit |
| Jupyter | Latest | Running notebooks | pip install jupyter |

pip install geopandas rasterio scikit-learn pandas numpy matplotlib xarray netcdf4 scipy fiona shapely streamlit

# 3. Data Acquisition

## 3.1 Sentinel-2

Go to browser.dataspace.copernicus.eu. Set: Collection=Sentinel-2, Product=S2MSI2A, Cloud<15%, Tile=T31UFU. Download 3-4 images per season across 2 years. Total ~20-25 images.

## 3.2 CBS

Go to pdok.nl, search 'CBS Wijken en Buurten 2024', download the GeoPackage.

## 3.3 Liander

Go to liander.nl/over-ons/open-data. Download: Opwekdata (PV counts), Terugleverdata (feed-in, column TOT_E_INV), Kleinverbruikgegevens (consumption, column TOT_E). All CSVs use semicolons.

Tip: Liander buurtcodes have trailing whitespace. Always apply .str.strip() before joining.

## 3.4 CM SAF Radiation

Go to data.eumetsat.int, search SARAH-3, filter SIS monthly mean. Download Dec 2023 to Feb 2026 (27 months). Extract .nc files into folders named by year-month.

# 4. SNAP Preprocessing (per image)

| Step | Menu Path | Settings |
| --- | --- | --- |
| 1. Open | File > Open Product | Select MTD_MSIL2A.xml |
| 2. RGB check | Right-click > Open RGB | Check Amsterdam for clouds |
| 3. Resample | Raster > Geometric > Resampling | Reference: B2, Method: Bilinear |
| 4. NDVI | Raster > Band Maths | (B8 - B4) / (B8 + B4) |
| 5. NDBI | Raster > Band Maths | (B11 - B8) / (B11 + B8) |
| 6. Cloud mask | Raster > Band Maths | if (scl_cloud_medium_proba ¦¦ scl_cloud_high_proba ¦¦ scl_cloud_shadow ¦¦ scl_thin_cirrus) then NaN else NDVI |
| 7. Subset | Raster > Subset | N:52.574 S:52.143 W:4.507 E:5.331 |
| 8. Export | File > Export > GeoTIFF | Name: S2_YYYYMMDD_amsterdam_processed.tif |

Tip: SNAP does NOT export virtual bands. Only raw bands (B2,B3,B4,B8,B11) survive. NDVI/NDBI are recomputed in Python.

Tip: Band order in exported GeoTIFF: Band1=B2, Band2=B3, Band3=B4, Band4=B8, Band5=B11. Descriptions are always None.

# 5. Running Pipeline 1 (Congestion)

Open congestion_analysis.ipynb. Run all cells top to bottom. Runtime: 30-60 minutes.

| Section | Time | Output |
| --- | --- | --- |
| Grid creation | <1 min | 2,217 cells |
| Satellite extraction | 15-30 min | NDVI/NDBI per cell |
| CBS + Liander joins | 2-5 min | Demographics + energy data |
| Target + RF training | 2-5 min | F1-macro 0.578 |
| Maps + export | <2 min | congestion_risk_grid.gpkg |

# 6. Running Pipeline 2 (Radiation)

Open radiation_estimation.ipynb. Run all cells. Runtime: 5-10 minutes. Requires Pipeline 1 output.

| Section | Time | Output |
| --- | --- | --- |
| Load congestion grid | <1 min | 2,217 cells from Pipeline 1 |
| Load SIS NetCDF | 1-2 min | 27 months radiation data |
| Comparison + drivers | <1 min | 223 consumption / 163 solar / 107 mixed |
| Export | <1 min | congestion_radiation_grid.gpkg |

# 7. Web Application

cd website/

streamlit run app.py

Opens at localhost:8501. Grid Analysis and Solar vs Grid pages show Nikola's work. Other pages are Anh's.

# 8. Troubleshooting

| Problem | Cause | Solution |
| --- | --- | --- |
| Band descriptions None | SNAP GeoTIFF export | Use band positions: 3=B4, 4=B8, 5=B11 |
| PV join returns 0 cells | Buurtcode whitespace | Apply .str.strip() to both columns |
| CBS shows -99995 | Missing data code | Replace with NaN |
| Liander CSV parse error | Semicolons, Dutch decimals | Use sep=';', decimal=',' |
| NetCDF empty after clip | Descending latitude | Swap slice direction |
| xarray not found | Not installed | pip install xarray netcdf4 |

# 9. Handover to Desislav

Key improvements for next semester:

1. Download tile T31UEU for full western Amsterdam coverage (currently 391 cells missing).

2. Download full BAG extract instead of WFS API (currently only 13 cells).

3. Contact Liander for spatial congestion declaration data to replace the proxy target.

4. Try XGBoost as alternative to Random Forest on the same features.

5. Add seasonal NDVI change rates as additional features.